| Chevron Phillips Chemical Company LP | Chevron Phillips Chemicals International N.V. | Chevron Phillips Chemicals Asia PTE Ltd. |
| --- | --- | --- |
| 10001 Six Pines Drive | Brusselsesteenweg 355 | 5 Temasek Boulevard |
| The Woodlands, TX 77380 | B-3090 Overijse, Belgium | 05-01 Suntec Tower Five |
| 800-858-4327 | +32 (0) 2 689 12 11 | Singapore 038985 |
| Technical Service: 832-813-4862 |  | +65 6337 9700 |

ASTM Subcommittee D02.01 on Combustion Characteristics

Cetane Number Calibration for Diesel Secondary Reference Fuels T-30 Blended with U-30 by Volume

| Vol % T-30 | Cetane Number | Vol % T-30 | Cetane Number | Vol % T-30 | Cetane Number |
| --- | --- | --- | --- | --- | --- |
| 0 | 20.3 | 34 | 38.9 | 68 | 57.4 |
| 1 | 20.9 | 35 | 39.4 | 69 | 58.0 |
| 2 | 21.4 | 36 | 40.0 | 70 | 58.5 |
| 3 | 22.0 | 37 | 40.5 | 71 | 59.1 |
| 4 | 22.5 | 38 | 41.1 | 72 | 59.6 |
| 5 | 23.1 | 39 | 41.6 | 73 | 60.1 |
| 6 | 23.6 | 40 | 42.1 | 74 | 60.7 |
| 7 | 24.1 | 41 | 42.7 | 75 | 61.2 |
| 8 | 24.7 | 42 | 43.2 | 76 | 61.8 |
| 9 | 25.2 | 43 | 43.8 | 77 | 62.3 |
| 10 | 25.8 | 44 | 44.3 | 78 | 62.9 |
| 11 | 26.3 | 45 | 44.9 | 79 | 63.4 |
| 12 | 26.9 | 46 | 45.4 | 80 | 64.0 |
| 13 | 27.4 | 47 | 46.0 | 81 | 64.5 |
| 14 | 28.0 | 48 | 46.5 | 82 | 65.0 |
| 15 | 28.5 | 49 | 47.1 | 83 | 65.6 |
| 16 | 29.1 | 50 | 47.6 | 84 | 66.1 |
| 17 | 29.6 | 51 | 48.1 | 85 | 66.7 |
| 18 | 30.1 | 52 | 48.7 | 86 | 67.2 |
| 19 | 30.7 | 53 | 49.2 | 87 | 67.8 |
| 20 | 31.2 | 54 | 49.8 | 88 | 68.3 |
| 21 | 31.8 | 55 | 50.3 | 89 | 68.9 |
| 22 | 32.3 | 56 | 50.9 | 90 | 69.4 |
| 23 | 32.9 | 57 | 51.4 | 91 | 70.0 |
| 24 | 33.4 | 58 | 52.0 | 92 | 70.5 |
| 25 | 34.0 | 59 | 52.5 | 93 | 71.0 |
| 26 | 34.5 | 60 | 53.1 | 94 | 71.6 |
| 27 | 35.1 | 61 | 53.6 | 95 | 72.1 |
| 28 | 35.6 | 62 | 54.1 | 96 | 72.7 |
| 29 | 36.1 | 63 | 54.7 | 97 | 73.2 |
| 30 | 36.7 | 64 | 55.2 | 98 | 73.8 |
| 31 | 37.2 | 65 | 55.8 | 99 | 74.3 |
| 32 | 37.8 | 66 | 56.3 | 100 | 74.9 |
| 33 | 38.3 | 67 | 56.9 |  |  |